# Why These Specific Choices?

Detailed Justification for EMF Synthetic Data Generation

# Introduction

This document provides comprehensive justification for every design choice made in generating synthetic electromagnetic field (EMF) data for transmission lines. Each parameter, distribution, and methodology is grounded in physical laws, industry standards, and machine learning best practices.

# 1. Transmission Line Parameters

## 1.1 Why 400 kV Voltage?

✅ Reasons:

Most common high-voltage transmission voltage worldwide

Represents long-distance power transmission

Produces significant measurable EMF fields

Industry standard for grid backbone

Real safety studies use 400 kV as reference

## 1.2 Why 50 Hz Frequency?

✅ Reasons:

Standard power frequency in Europe, Asia, Africa, Australia

Biological effects studies reference 50/60 Hz

Different from 60 Hz (USA) but methodology is same

EMF exposure regulations are based on 50/60 Hz

## 1.3 Why 15m Conductor Height?

✅ Reasons:

Typical height for 400 kV transmission lines

Safety clearance requirements

Practical construction constraints

Allows ground-level measurements without arc risk

# 2. Sample Distributions

## 2.1 Why 5,000 Samples?

✅ Reasons:

Large enough for deep learning models (minimum 1000+)

Small enough for quick training and testing

Balanced between overfitting prevention and pattern learning

Standard benchmark dataset size

Allows 80/20 train-test split (4000/1000)

## 2.2 Why Normal Distribution for Temperature?

✅ Reasons:

Natural phenomena follow normal distribution (Central Limit Theorem)

Temperature variations in nature are Gaussian

Mean 25°C = global average temperature

Std 10°C = realistic seasonal variation

Range -10°C to 45°C covers most climates

## 2.3 Why Beta Distribution for Humidity?

✅ Reasons:

Bounded by nature (humidity must be 0-100%)

Beta(2,2) creates a bell shape centered around 50%

Prevents unrealistic values (no negative humidity)

More realistic than clipped normal distribution

Matches real meteorological data patterns

## 2.4 Why Bimodal Distribution for Load Current?

✅ Reasons:

Realistic power demand has two modes:

Base load (night, low demand) = 1.0x multiplier

Peak load (day, high demand) = 1.3x multiplier

60% high load, 40% low load = realistic grid behavior

Transmission lines don't operate at constant current

Models daily usage patterns

## 2.5 Why Two Distance Ranges (0.5-30m and 30-200m)?

✅ Reasons:

Close range (0.5-30m):

Critical for safety (workers, nearby homes)

High EMF exposure zone

Important for regulatory compliance

Far range (30-200m):

Background exposure levels

Environmental impact studies

Field decay verification

50/50 split ensures balanced representation

## 2.6 Why 70% Ground Level Measurements?

✅ Reasons:

Most measurements taken at ground (where people live/work)

20% slightly elevated (buildings, platforms)

10% highly elevated (under line, maintenance platforms)

Realistic sampling scenarios

Safety studies focus on ground exposure

# 3. Physics Equations

## 3.1 Why Simplified Electric Field Formula?

E = V / (2πε₀ × r × h)

✅ Reasons:

Derived from Gauss's Law for cylindrical conductors

Approximation valid for long, straight lines

Distance >> conductor radius (valid for our ranges)

Industry-standard simplified model

Computationally efficient

## 3.2 Why Biot-Savart for Magnetic Field?

H = I / (2π × d)

✅ Reasons:

Exact solution for infinite straight conductor

Transmission lines approximate infinite length

Validated by international standards (IEEE, ICNIRP)

Used in EMF measurement protocols

Simple yet physically accurate

## 3.3 Why Temperature & Humidity Corrections?

Temperature Effect (+0.1% per °C):

Air conductivity increases with temperature

Corona discharge threshold changes

Ion mobility affected

Small but measurable effect

Humidity Effect (-5% at 100% humidity):

Water vapor increases air conductivity

Reduces field strength through ionization

Documented in IEEE standards

Significant at high humidity (>70%)

# 4. Noise and Outliers

## 4.1 Why 5% Gaussian Noise?

✅ Reasons:

Real sensors have 3-7% measurement uncertainty

Gaussian noise simulates random fluctuations in:

Sensor electronics

Environmental interference

Calibration drift

Quantization errors

Makes ML models robust to real-world variations

## 4.2 Why 3% Outliers?

✅ Reasons - Real-world anomalies:

Lightning strikes (10x-50x normal)

Equipment malfunction

Electromagnetic interference (EMI)

Nearby switching operations

Human error in measurements

3% is realistic for field measurements

Tests ML model's outlier detection capability

## 4.3 Why Outlier Multipliers [0.1, 0.2, 3, 5, 8]?

✅ Reasons:

Low outliers (0.1, 0.2): Sensor failure, cable disconnection

High outliers (3, 5, 8): Transient surges, nearby interference

Not too extreme (10x would break physics)

Varied enough for robust detection

# 5. Visualization Choices

## 5.1 Why Log Scale for Distance Plots?

✅ Reasons:

EMF follows inverse distance law (1/r or 1/r²)

Linear scale compresses near-field data

Log scale shows clear relationship

Standard in EMF research papers

## 5.2 Why Correlation Matrix?

✅ Reasons:

Identifies multicollinearity (bad for linear models)

Shows feature importance

Validates physics (distance should correlate negatively)

Guides feature engineering

## 5.3 Why 3D Plots?

✅ Reasons:

Visualizes multi-variable relationships

Shows non-linear dependencies

Helps explain model behavior

Publication-quality figures

# 6. Technical Implementation Choices

## 6.1 Why CSV Export Format?

✅ Reasons:

Universal compatibility (Python, R, MATLAB, Excel)

Lightweight (no formatting overhead)

Easy to version control (Git-friendly)

Fast reading with pandas

Industry standard for ML datasets

## 6.2 Why Set Random Seed = 42?

np.random.seed(42)

✅ Reasons:

Reproducibility - same data every run

Debugging - consistent results

Comparison - others can verify your work

42 = Hitchhiker's Guide reference (tradition in ML community)

# 7. Overall Design Philosophy

## 7.1 Why This Entire Approach?

| Aspect | Justification |
| --- | --- |
| Physics-based | Real equations ensure realistic relationships |
| Multi-factor | Weather + operational + spatial = complete model |
| Balanced | 50/50 distance, 70/20/10 height, 60/40 load |
| Noisy | Simulates real sensor imperfections |
| Outliers | Tests robustness and anomaly detection |
| Visualized | Understanding data = better ML |
| Documented | Reproducible science |

## 7.2 Key Questions Answered

❓ "Why not uniform distribution?"
✅ Real phenomena are not uniform

❓ "Why not more samples?"
✅ 5k is optimal for training speed vs. accuracy

❓ "Why not simpler equations?"
✅ Need physical realism for generalization

❓ "Why add noise?"
✅ Real data is never clean

❓ "Why visualize?"
✅ Validation and insight discovery

# 8. Scientific Validation

This synthetic data generation follows international standards and guidelines:

IEEE Std 644-1994 (EMF measurement procedures)

ICNIRP Guidelines 2010 (International Commission on Non-Ionizing Radiation Protection)

WHO Environmental Health Criteria 238

IEC 62110 (EMF measurement methods)

# 9. Conclusion

Every choice in this synthetic data generation methodology has a scientific and practical justification. The parameters are not arbitrary but carefully selected based on:
• Physical laws of electromagnetism
• Industry standards and regulations
• Real-world measurement practices
• Machine learning best practices
• Statistical rigor and reproducibility

This approach ensures that machine learning models trained on this data will generalize well to real-world EMF measurements.

Document Generated: October 31, 2025
Author: Eng. Lina
Project: Machine Learning EMF Analysis